Table of Contents

1. Purpose	2

2. Scope	2

3. Prerequisites	3

4. Definitions	3

5. Troubleshooting Steps for Common Access Point Issues	3

Issue 1: No Connectivity to the AP	3

Issue 2: Poor Wi-Fi Signal or Coverage	4

Issue 3: Slow Wi-Fi Performance or Intermittent Connectivity	5

Issue 4: Authentication Failures (WPA/WPA2/WPA3)	6

Issue 5: AP Not Broadcasting SSID / SSID Visibility Issues	7

Issue 6: Rogue AP Detection or Unauthorized Devices	7

Issue 7: Firmware or Hardware Issues	8

6. Conclusion	8

SOP Troubleshooting Access Point (AP) Connectivity and Performance Issues

# 1. Purpose

This document provides a Standard Operating Procedure (SOP) for diagnosing and resolving a wide range of issues related to Access Points (APs) in an enterprise wireless network environment. The goal is to outline detailed troubleshooting steps for various AP issues, including connectivity problems, performance degradation, signal issues, authentication failures, and configuration errors, ensuring minimal downtime and restoring optimal network functionality.

# 2. Scope

This SOP covers all potential issues that may arise with Access Points (APs) in a wireless network, including but not limited to:

- No connectivity to the AP.

- Slow performance or intermittent connectivity.

- Poor signal quality or coverage issues.

- Authentication failures (WPA2/WPA3 issues).

- AP not broadcasting SSID or SSID visibility issues.

- Security-related issues (e.g., MAC filtering, rogue AP detection).

- Firmware and hardware issues.

- DHCP issues and IP assignment failures.

The procedures and steps provided here apply to common enterprise-grade Access Points from manufacturers such as Cisco, Aruba, TP-Link, and Ruckus.

# 3. Prerequisites

Before troubleshooting, ensure the following:

- Access to the AP management interface (web interface or CLI).

- End user details, including device type (e.g., laptop, smartphone), operating system, and any error messages displayed.

- Access to AP and wireless controller configurations (if applicable).

- Basic understanding of wireless networking, including SSIDs, channels, security protocols (WPA/WPA2/WPA3), and wireless interference.

- Firmware version and configuration details for the AP.

# 4. Definitions

- SSID (Service Set Identifier): The name assigned to a wireless network.

- WPA/WPA2/WPA3: Security protocols for wireless networks.

- RSSI (Received Signal Strength Indicator): A measurement of signal strength between the device and the AP.

- Interference: External factors (such as other wireless devices) that degrade Wi-Fi performance.

- AP Controller: A centralized device used to manage multiple Access Points in a network (e.g., Cisco Wireless LAN Controller).

- DHCP (Dynamic Host Configuration Protocol): A protocol that automatically assigns IP addresses to devices on the network.

- Channel Overlap: When two or more wireless networks are operating on the same or overlapping channels, causing interference.

# 5. Troubleshooting Steps for Common Access Point Issues

## Issue 1: No Connectivity to the AP

Symptoms:

- User cannot connect to Wi-Fi at all.

- Device doesn’t see the SSID.

- Device sees the SSID but cannot connect.

Step 1: Verify AP Power and Physical Status

- Ensure the AP is powered on.

- Check LED indicators to confirm normal operation.

  - Solid Green: AP is functioning properly.

  - Flashing Red: Indicates an error, such as network failure or hardware issue.

  - No LED: AP may be powered off or experiencing hardware failure.

Step 2: Confirm AP is Broadcasting SSID

- Log into the AP management interface.

- Check if the SSID is active and set to broadcast.

- If the SSID is hidden, confirm the network name and ensure the user is aware of it.

Step 3: Check Channel Settings

- Ensure that the AP is not on a crowded or overlapping channel (particularly in the 2.4 GHz band).

- Switch channels if necessary to avoid interference with neighbouring APs.

Step 4: Verify Network Configuration

- Ensure the AP is properly connected to the wired network and has a valid IP address.

- Check for any network issues such as DHCP assignment failures or cabling problems.

Step 5: Reboot AP

- Sometimes, a simple reboot can restore connectivity.

## Issue 2: Poor Wi-Fi Signal or Coverage

Symptoms:

- Wi-Fi signal is weak or inconsistent.

- Users experience dropouts when moving around.

Step 1: Check Signal Strength (RSSI)

- Use a Wi-Fi analyser to measure the signal strength of the AP.

  - Ideal signal strength should be greater than -67 dBm for general usage.

  - Anything less than -80 dBm may lead to poor performance.

Step 2: Evaluate AP Placement

- Ensure the AP is placed in a central location to provide optimal coverage.

- Avoid physical obstructions (walls, floors, metal objects) that can attenuate the signal.

Step 3: Adjust Transmit Power

- Log into the AP management interface and adjust the transmit power.

  - If coverage is limited, consider increasing the transmit power (within safe limits).

  - If coverage is too wide and causing interference, decrease the transmit power.

Step 4: Identify Interference Sources

- Check for electromagnetic interference from devices like microwaves, cordless phones, or Bluetooth devices.

- Use a Wi-Fi spectrum analyser to identify other nearby networks operating on the same channel.

Step 5: Channel Adjustment

- If the AP is using the 2.4 GHz band, switch to a less congested channel (channels 1, 6, or 11).

- If available, enable 5 GHz and use channels that are less crowded.

## Issue 3: Slow Wi-Fi Performance or Intermittent Connectivity

Symptoms:

- Users experience slow speeds.

- Frequent disconnects from the network.

Step 1: Verify AP Load

- Check how many clients are connected to the AP. If the AP is overloaded, disconnect some clients or move them to another AP.

Step 2: Check for Channel Interference

- Use a Wi-Fi analyser to check for channel overlap and adjust the AP’s channel settings to avoid interference.

Step 3: Evaluate Network Bandwidth

- Check network throughput by running a speed test from a wired client to ensure the internet connection isn’t the bottleneck.

- If network performance is poor, ensure that QoS (Quality of Service) is properly configured to prioritize critical traffic.

Step 4: Check for Firmware Issues

- Ensure that the AP is running the latest firmware version.

- Update the firmware if there are any known issues related to performance or connectivity.

Step 5: Check for DHCP/IP Address Conflicts

- Ensure that there are no IP address conflicts on the network.

- Verify that DHCP servers are functioning correctly and allocating IP addresses to clients.

## Issue 4: Authentication Failures (WPA/WPA2/WPA3)

Symptoms:

- Users cannot authenticate to the network.

- Devices prompt for credentials repeatedly.

Step 1: Check Security Settings

- Ensure that the WPA/WPA2/WPA3 settings on the AP match the security settings on the client devices.

- Confirm that the correct passphrase is entered on the client device.

Step 2: Review RADIUS Server Configuration (if applicable)

- If using 802.1X authentication, verify that the RADIUS server is reachable and responding.

- Check RADIUS logs for any authentication failures or misconfigurations.

Step 3: Verify MAC Address Filtering

- If MAC address filtering is enabled, ensure that the MAC address of the client device is allowed.

- Disable MAC filtering if it’s unnecessary for security purposes.

## Issue 5: AP Not Broadcasting SSID / SSID Visibility Issues

Symptoms:

- SSID is not visible when scanning for available networks.

- Devices cannot find the network.

Step 1: Verify SSID Settings

- Ensure that the SSID broadcast is enabled in the AP configuration.

- Ensure that the SSID is not hidden (if it needs to be visible).

Step 2: Check AP Radio Settings

- Make sure the AP radios (2.4 GHz or 5 GHz) are configured correctly.

- Ensure the AP is not configured to disable specific bands or hide radios.

Step 3: Restart the AP

- Sometimes the AP may fail to broadcast SSID due to a temporary issue; a reboot may resolve it.

## Issue 6: Rogue AP Detection or Unauthorized Devices

Symptoms:

- A rogue AP is detected within the network, potentially compromising security.

Step 1: Identify Rogue APs

- Use Wi-Fi scanning tools to detect unauthorized or rogue APs.

- Check the AP’s management interface for any anomalies or unauthorized devices.

Step 2: Disable or Isolate Rogue AP

- Disable or block the rogue AP immediately to prevent unauthorized access.

- Reconfigure the AP’s security settings (e.g., change WPA keys or increase authentication strength).

Step 3: Review Security Logs

- Check for any security breaches or attempts to connect to rogue APs.

- Perform a security audit to identify any potential weaknesses in the wireless network configuration.

## Issue 7: Firmware or Hardware Issues

Symptoms:

- The AP is unresponsive or has malfunctioning hardware.

- Firmware bugs causing instability or connectivity issues.

Step 1: Check for Firmware Updates

- Verify the current firmware version.

- Update the firmware to the latest stable version.

Step 2: Hardware Check

- Ensure the AP has no physical damage (e.g., broken antenna).

- Check for signs of hardware failure, such as overheating or power issues.

Step 3: Reset to Factory Defaults

- If all else fails, consider performing a factory reset on the AP and reconfigure it.

# 6. Conclusion

This SOP provides a comprehensive guide to troubleshooting all common Access Point (AP) related issues in a network environment. By following the steps outlined in this document, we can efficiently diagnose and resolve issues such as connectivity problems, performance degradation, authentication errors, and hardware malfunctions, ensuring uninterrupted and secure wireless access for end users.